Standard Operating Procedure for

Controlled Burning Operations

| What it is being used for/why is activity needed | Bonfires and controlled burning of vegetation |
| --- | --- |
| What are the hazards | - Burns to individuals - Fire gets out of control leading to wildfire or property damage - Damage to utilities causing major incident (eg gas explosion) |
| What is the process you have to follow to complete the job safely | - Check weather and site conditions: Do not have a fire in extremely dry conditions - Choose suitable location for the fire site to avoid accidental spread or unnecessary damage - Do not have fires within 100m of overhead powerlines or underground services without a further more detailed site and activity risk assessment. The risk of arcing due to overhead power lines is greater in wet or foggy weather - Do not have fires on sites with buried waste e.g. ex-landfill or industrial sites - Avoid having fires close to other properties or road – especially downwind – do not allow smoke to drift across roads - Avoid wearing polyester or other man-made fibre clothing when working near a fire - Limit number of people working around the fire to a safe and manageable number of experienced workers - Keep fire under control at all times - Leave fire safe and not at risk of spreading at the end of the task. Tape or fence off area if risk of public/dogs walking on hot embers |
| What training and supervision is required | Internal competency training from experience officer |
| What PPE is required | Gloves |
| What other equipment is needed | - Sufficient water and/or fire beaters to manage risk of spreading and to deal with serious burns - Carry burns kit in addition to normal first aid kit |
| What do you do if something goes wrong | Call 999 Fire and Rescue Service to request assistance |
| Where can you get more information | Utilities information for underground/overhead services should be shown on Site Risk Assessment. Advice available from individual utilities companies. |
| Name of person completing this form | Esther Clarke |
| Date form completed/updated | 19/06/2023 |