Standard Operating Procedure for

Powered hedge trimmer inc. pole mounted

| What it is being used for/why is activity needed | To cut back overgrown vegetation or hedges. To prevent encroachment of unwanted vegetation or sight lines |
| --- | --- |
| What are the hazards | - Hand arm vibration to user during operation - Noise during operation - Laceration Injury from trimmer head to user and those nearby whilst working - Musculoskeletal injury from manual handling of pole trimmer at arm’s length. - Cut overhead debris landing on operator - Fuel spillage |
| What is the process you have to follow to complete the job safely | - Carry out checks of machine prior to use including on/off switch, dead man’s handle (if fitted), battery is properly located and that trimmer blade is undamaged and lubricate grease nipple (if fitted) and blade itself with WD40 correctly. - Ensure machine has sufficient battery power - Before cutting check that trimer head is running freely and is correctly adjusted i.e. bolt tight etc. - Reduce heavy vegetation back in short lengths with several passes if possible to reduce clogging - Do not over reach with the pole trimmer and maintain both feet firmly on the ground for stability. - Do not try and cut vegetation of a larger diameter than the trimmer can cope with or head may jam. (often this is specified by manufacturer) - Try and avoid cutting directly overhead and be aware of where arisings will likely to fall. - Other people to maintain safe distance - Wear the harness provided as this reduces the strain on arms whilst working (pole trimmer only). - If cutting hedges be mindful of any wire fencing materials - Take rest if needed |
| What training and supervision is required | In house training and competency check from experienced user. Supervision from experience user until new user is also sufficiently experienced. |
| What PPE is required | Gloves to dampen vibration and protect hands from vegetation Forestry helmet to protect ears, eyes and head from falling arisings etc. Work boots to protect feet from falling debris etc. |
| What other equipment is needed | Correct harness for the pole trimmer, lubrication and tools to tighten nuts on cutting head if needed Fuel cans if petrol powered machine and basic tool kit. Spill Kit |
| What do you do if something goes wrong | Stop the machine. Deal with incident as required. Report incident. Repair machine as required. |
| Where can you get more information | Manufacturers User manual Manufacturers website |
| Name of person completing this form | Toby Spicer |
| Date form completed/updated | 30/06/2025 |